ОЦІНКА ПОТРЕБ У ДОСЛІДЖЕННЯХ РОБОТИ АВТОБАЛАНСУЮЧИХ ПРИСТРОЇВ З РІДИННИМИ РОБОЧИМИ ТІЛАМИ
Кручинін І.М. 1, Драч І. В. 2
1, 2 Хмельницький національний університет, вул. Інститутська, 11, Україна
E-mail:1 illya.kruchynin@kmgas.com.ua, 2 cogitare410@gmail.com
Для усунення дисбалансу тіла, що обертається, використовують рідинні автобаласуючі пристрої (АБП) у вигляді порожнистої камери з рідкими робочими тілами. Вони є стандартною функцією у більшості побутових пральних машин, але також використовуються у важких промислових обертових машинах. До прикладу, для пральних машин використовуються рідинні стабілізатори, які складаються з порожнистого кільця, подібного до гумового обруча але, зазвичай, з прямокутним перерізом, що містить невелику кількість рідини. Кільце, як правило, кріпиться над барабаном. Рідина в кільці вільно переміщається і прагне поєднати головну центральну вісь інерції ротора з його віссю обертання.

Основна ідея з'явилася вже в 1912 році, а патент США був наданий в 1916 році (Леблан, 1916 [1]), де і було викладено робочий принцип рідинного балансира. А саме: коли машина обертається при високій кутовій швидкості Ω, рідина утворює тонкий шар на внутрішній поверхні зовнішньої стінки, зображений на рис. 1 [1].
[image: image1.jpg]Q>Q,

\Q <q, \





а 


б

Рис. 1. Принцип роботи рідинного стабілізатора за Лебланом

Розглядається ситуація, коли присутня незбалансована маса m, наприклад, через нерівномірність розподілу одягу в пральній машині. Ротор має критичну кутову швидкість Ωcr, де відцентрові сили зрівноважуються силами, зумовленими відновлюваними пружинами. Нижче цієї швидкості (Ω < Ωcr) центр маси рідини буде розташовуватися «на тій самій стороні», що і незбалансована маса, як показано на рисунку 1 а (тут M позначає масу порожнього ротора, а M - маса рідини, що міститься в камері). При певній надкритичній кутовій швидкості Ω > Ωcr (скажімо, під час процесу сушки) центр маси рідини переміститься на "протилежну сторону" відносно незбалансованої маси, як показано на рисунку 1 б, в результаті чого зменшиться "дисбаланс маси" і, таким чином, зменшаться відцентрові сили і зменшаться амплітуди коливань ротора.

Оригінальний макет, наведений у патенті [1] складався з одного або декількох дуже вузьких концентричних каналів (вузьких у радіальному напрямку, але широких в осьовому напрямку, тобто перпендикулярно до площини, зображеної на рис.1, частково заповненого "рідиною або дуже малими сталевими кульками або металевими дробинками" [1]. Сирл (1932) [22] розглядав і піддав критиці рідинний балансир Леблана; а пізніше і Ден Гартог (1985) підтримав погляди Сирла. Тут йдеться про те, що балансир Леблана не може працювати з рідиною, а лише із сталевими кульками, і, таким чином, винахід не був визнаним. Імовірно, це пов'язано з дуже вузькими каналами, які в основному запобігають утворенню поверхневих хвиль.

Тим не менш, автоматична пральна машина, оснащена рідинним балансиром, була представлена в 1940 році і запатентована в 1945 році (Дайєр, 1945) [2]. Макет рідинного балансира був дуже схожий на сучасні макети, з широкою концентричною камерою, досить широкою, щоб надати можливість утворенню поверхневій хвилі з великими амплітудами.

Ідея, таким чином, не нова; але останнім часом спостерігається відроджений інтерес до неї як у промисловості, так і в наукових сферах. 
Слід підкреслити, що вперше зробив спробу теоретично обґрунтувати принцип дії АБП Леблана, кільцевих, маятникових і кульових АБП у циклах своїх статей Сирл. Він запропонував плоску модель ротора і АБП. У її рамках у ротора існує єдина критична швидкість, при перевищенні якої ротор починає обертатися легкою стороною назовні і починає проявлятися явище самоцентрування ротора, яке і було покладено Сирлем в основу принципу роботи усіх пасивних АБП. Насправді, тут явище самоцентрування видається за явище автобалансування рідиною. Підходи і результати робіт Сирла склали основу наступних досліджень в галузі балансування роторів машин пасивними АБП. А отриманий без врахування специфічних властивостей рідни, сил опору, а відтак, теоретично необґрунтований і експериментально неперевірений висновок про обмеженість діапазону працездатності таких автобалансуючих пристроїв закритичною (або зарезонансною) зоною обертання ротора є суперечливим і до тепер. Отже, залишається необхідним більш детальне вивчення цього явища.
Підсумок огляду літератури з досліджень рідинних автобалансирів представлений в таблиці 1 у вигляді літературного картографування. Публікації, які містять більш повні результати теоретичних та експериментальних досліджень, пов'язані з гнучкими роторами, позначені зеленим кольором (+). Жовті області (±) допомагають визначити поточні потреби в дослідженнях. 
Таблиця 1. 
Картографування публікацій
	Публікації
	Експериментальні дослідження
	Чисельне моделювання
	Аналітичні дослідження
	Стабільність роботи АБП
	Взаємодія рідина-ротор
	
	Публікації
	Експериментальні дослідження
	Чисельне моделювання
	Аналітичні дослідження
	Стабільність роботи АБП
	Взаємодія рідина-ротор

	1
	+
	±
	±
	-
	±
	
	22
	+
	±
	±
	-
	±

	2
	+
	±
	±
	-
	±
	
	23
	-
	±
	±
	-
	±

	3
	+
	±
	±
	-
	±
	
	24
	-
	±
	±
	-
	±

	4
	+
	±
	±
	-
	±
	
	25
	-
	±
	±
	-
	±

	5
	+
	±
	±
	-
	±
	
	26
	-
	±
	±
	-
	±

	6
	+
	±
	+
	-
	±
	
	27
	-
	±
	±
	-
	±

	7
	+
	+
	±
	-
	±
	
	28
	-
	±
	±
	-
	±

	8
	+
	+
	±
	-
	±
	
	29
	-
	±
	±
	-
	±

	9
	+
	±
	+
	-
	±
	
	30
	-
	±
	±
	-
	±

	10
	-
	±
	+
	-
	±
	
	31
	-
	±
	±
	-
	±

	11
	+
	±
	+
	-
	±
	
	32
	+
	±
	+
	±
	±

	12
	+
	±
	±
	-
	±
	
	33
	-
	±
	+
	±
	±

	13
	-
	±
	±
	+
	±
	
	34
	-
	+
	+
	+
	±

	14
	-
	±
	±
	+
	±
	
	35
	±
	±
	+
	+
	±

	15
	-
	±
	±
	+
	±
	
	36
	-
	±
	±
	±
	±

	16
	-
	±
	±
	+
	±
	
	37
	-
	±
	+
	±
	±

	17
	-
	±
	±
	+
	±
	
	38
	-
	±
	+
	+
	+

	18
	-
	±
	±
	+
	±
	
	39
	-
	±
	+
	+
	+

	19
	-
	±
	±
	+
	±
	
	40
	-
	±
	±
	±
	±

	20
	-
	±
	±
	+
	±
	
	41
	-
	±
	+
	+
	+

	21
	-
	±
	±
	+
	±
	
	42
	+
	±
	±
	±
	±


Отже, існує доцільність досліджень у розумінні реакції дисбалансу, взаємодії рідини та ротора для підвищення стабільності руху ротора та більш ефективних механізмів балансування гнучких роторів з аналітичним підходом. Однак, існує також потреба у числових експериментах та візуальному моделюванні роботи рідинного АБП.

Література

1. LeBlanc, M., 1914, “Automatic Balancer for Rotating Bodies,” US Patent 1,159,052.

2. Dyer, J., 1945,“Domestic Appliance,” US Patent 2,375,635.

3. De Remer, J. G., 1950,“Blancing of Rotatable Bodies,” US Patent 2,525,781.

4. Van Kirk, J. y Burmeister, L., "An automatic balancer design for a vertical-axis clothes washing machine," ASME Paper No. 76-DE-24, 1976.

5. Hayashi, G., Yamamoto, K., Yamamoto, Y., 1977, “Balancing Ring of Centrifugal Extractor ,” US Patent 4,044,626.

6. Conrad, D.C., 1994, “On The Fundamentals of Automatic Washing Machine Design Based Upon Dynamic Constraints”, Ph.D. Thesis, Purdue University.

7. Hoon, H. S., Young. L. J., Suzuki, S., and Gu, H. W., 2001, “A Study on the Dynamic Behaviour of an Automatic Washing Machine”, Korea ADAMS User Conference, 11 (8-9), 1-6.

8. Morio, M., and Yutaka, N., 2002, “Washing Machine Dehydration Dynamics Analysis,” Nihon Kikai Gakkai Nenji Taikai Koen Ronbunshu, 5, 209-216.

9. Bae, S. Lee, J. M., Kang, J. S. and Yun, J.R., 2002, “Dynamic Analysis of an Automatic Washing Machine with a Hydraulic Balancer,” Journal of Sound and Vibration, 257, 3-18.

10. Urbiola, L., 2005, “Cálculo de los Efectos Inerciales (Radiales y Tangenciales) del Fluido del Aro de Balance,” Internal Research Report, mabe TyP.

11. Jung. C.-H., Kim, C.-S., and Choi, Y.-H., 2008, “A Dynamic Model and Numerical Study on The Liquid Balancer Used in an Automatic Washing Machine,” Journal of Mechanical Science and Technology, 22, 1843-1852.

12. Urbiola, L., et. al., 2009, “Hydrodynamic Balance Ring for Centrifugal Rotation Machines,” US Patent and Trademark Office Application 20090158783 A1.

13. Newkirk, B. L., 1924, “Shaft Whipping,” General Electric Review, 27(3), 169- 178.

14. Kimball, A. L. Jr., 1924, “Internal Friction Theory of Shaft Whirling,” General Electric Review, 27(4), 244-251.

15. Gunter, E. J., 1966, “Dynamic Stability of Rotor-Bearing Systems,” NASA Technical Report, SP-113.

16. Black, H.F., 1976, “The Stabilizing Capacity of Bearings for Flexible Rotors with Hysteresis,” ASME Journal of Engineering for Industry, 2, 87-91.

17. Lund, J. W., 1986, “Destabilization of Rotors from Friction in Internal Joints with Micro-slip,” International Conference in Rotordynamics, JSME, 487-491.

18. Srinivasan, A., 2003, “The Influence of Internal Friction on Rotordynamic Instability,” M.S. Thesis, Texas A&M University, May.

19. Kollmann, F. G., 1962, “Experimentelle und Theoretische Untersuchungen Uber die Kritischen Drehzahlen Flussigkeitsgefullter Hohlkorper,” Forsch. Geb. Ingenieurwes, 28, pp. 115–123; 1962, Forsch. Geb. Ingenieurwes, 28, pp. 147– 153.

20. Ehrich, E. F., 1967, “The Influence of Trapped Fluids on High Speed Rotor Vibration,” ASME J. Eng. Ind., 89, pp. 806–812.

21. Kirk, R. G., Mondy, M. E., and Murphy, R. C., 1984, “Theory and Guidelines to Proper Coupling Design for Rotor Dynamic Considerations,” ASME J. Vib. Acoust., Stress, Reliab. Des., 106, pp. 129–138.

22. Thearle E., 1932, “A New Type of Dynamic-Balancing Machine,” Trans. ASME 54, APM-54-12, pp. 131-141.

23. Sperling, L., Ryzhik, B. y Duckstein, H., 2001, “Two-Plane Automatic Balancing,” Machine Dynamics Problems, Proceedings of 7th Polish German Workshop on Dynamical Problems in Mechanical Systems, pp. 139-152.

24. Sperling, L., Ryzhik, B., Linz, C., and Duckstein, H., 2002, “Simulation of Two- Plane Automatic Balancing of a Rigid Rotor”, Mathemathics and Computers in Simulation, 58, pp. 351-365.

25. Green, K., Champneys, A. R., y Lieven N. J., 2006, ”Bifurcation Analysis of an Automatic Dynamic Balancing Mechanism for Eccentric Rotors,” Journal of Sound and Vibration, 291, pp. 861-881.

26. Green, K., Champneys, A. R., y Friswell M. I., 2006, “Analysis of the Transient Response of an Automatic Dynamic Balancer for Eccentric Rotors,”  International Journal of Mechanical Sciences, 48, pp. 274-293.

27. Horvath, R., Flowers, G., Fausz, J., 2006, “Passive Balancing for Rotor Systems,” Ph.D. thesis, Auburn University, Alabama.

28. Tatsumi, H., and Ito, M., 1984, “Balancer For Use in Centrifugal Rotary Machine,” US Patent 4,433,592.

29. Lee, J. Y., 1998, “Equilibrador de Esfera para Máquina de Lavar,” República Federativa de Brasil, PI9703376-6A.

30. Southworth, D.W., 2003, “Dynamic Balancer for An Automatic Washer,” US Patent 6,658,902 B2.

31. Kim, J.-S., 1998, “Balancing Device for Drum Washing Machine,” US Patent 5,802,885.

32. Urbiola-Soto, L., Lopez-Parra, M., 2010, “Experimental and Analytical Investigation on a Liquid Balance Ring for Automatic Washing Machines”, Proceedings of the ASME 2010 10th Biennial Conference on Engineering Systems Design and Analysis (ESDA 2010), Istanbul, Turkey, July 12-14.

33. Urbiola-Soto, L., and Lopez-Parra, M., 2011, “Dynamic Performance Of The LeBlanc Balancer For Automatic Washing Machines,” ASME Journal of Vibration and Acoustics, 133-4 (2011).

34. Urbiola-Soto, L., and Lopez-Parra, M., 2011, “Stability of Flexible Rotors with a LeBlanc Balancer,” ASME paper GT2011-46848, Proceedings of AME Turbo Expo 2011, Vancouver, Canada, June 6-10.

35. Urbiola-Soto, L. and Lopez-Parra, M., 2013, “Liquid Self-Balancing Device Effects on Flexible Rotor Stability”, Shock and Vibration, Volume 20, Issue 1, Pages 109-121.
36. Pashkov, E.N., Martyushev, N. V., Masson I. A., 2014, “Evaluation of Gravitational Force Effect on Balancing Processes in Liquid-Type Autobalancing Devices”, Advanced Materials Research : Scientific Journal. - 2014. - Vol. 1040 : High Technology: Research and Applications 2014 (HTRA 2014). - P. 642-649. 
37. Majewski, T., 2010, “Fluid balancer for a washing machine“, In Proceedings of the XVI Intemational Congress 2010. Pages 1-10. SOMIM (Society of Mechanical Engineers of Mexico).

38. Hendricks, S.L., Morton, J.B., 1979, “Stability of a rotor partially filled with a viscous incompressible fluid“, J. Appl. Mech. – 1979. Vol. – 46. P. - 913-918.

39. Holm-Christensen, O., Trаäger, K.A., 1991, “Note on rotor instability caused by liquid motions“, J. Appl. Mech., 1991. Vol. – 58. P. - 804-811.

40. Cunico, M.W.M., 2015, “Characterization and Modelling of LeBlanc Hydrodynamic Stabilizer: A Novel Approach for Steady and Transient State Models“, Modelling and Simulation in Engineering. Volume 2015 (2015).
41. Langthjema, M. A., Nakamurab, T., 2013, “Dynamics of the fluid balancer: Perturbation solution of a forced Korteweg-de Vries-Burgers equation“, RIMS, Kyoto University, V. 1847, 73-85.
42. Nakamura, T., 2009, “Study on the improvement of the fluid balancer of washing machines“, In Proceedings of the 13th Asia-Pacific Vibrations Conference, 22-25 November 2009, pages 1-8. University of Canterbury, New Zealand.

